PE 7023 	Spring 2026

Advanced Production Engineering 	Design Project

Cem Sarica

# Advanced Production Generic Project

“Big Oil” company (BO) is studying the development of a “New Field” (NF) recently discovered.  Your team of production engineers is requested to design the production facilities and generate a production profile for the next 30 years for a new field under consideration.  The maximum rate is 45,000 liquid STB/d.  It is also required to complete a specific study; the topics are listed at the end of the project statement. Part 1 accounts for 70% of the grade, and Part 2 accounts for 30%.

In reality, production engineers and reservoir engineers work closely by combining their models to account for all relevant factors and build an appropriate production profile.  In this project, you will focus on the production side.  All assumptions from the reservoir side are simplified and treated as accurate.  You are required to use PIPESIM.

## Reservoir data

(See Excel sheet with data on the fluid and reservoir)

Assumptions

- The reservoir has an initial gas cap.  Therefore, production above the bubble point is impractical.

- The oil column is at the bubble point.  The oil composition is assumed to be constant throughout the field life.

- The production mechanism will be lateral water flooding, which maintains the reservoir pressure at the bubble point.

- The well spacing is homogeneous.

- You can neglect the wellbore deviation effect and all geometry effects on the inflow performance.

- You can neglect perforation effects.

- If you think some data is missing, make reasonable assumptions.

## Geometries

Well placements, well trajectories, and bathymetries of subsea piping are provided.

(See Excel sheet geometry data)

Part 1

### I. Well Design:

The production wells have not been drilled yet.  You will size the tubing and production casing for the drilling engineers.

Reservoir engineers assure you that the inflow can be modeled with the single-phase steady-state solution of the Darcy equation for a vertical fully penetrating well:

PI=qliqp-pwf=kh141.2Boμolnrerwf-0.5+S

The drilling and completion costs are highly sensitive to the wellbore radius, rwf.  You would like to find the minimum tubing size that allows you to produce the maximum flow rates for each development scenario.

1.- Plot the productivity index PI as a function of rwf.  Comment on your results.

2.- Assume that the tubing ID is 0.8 rwf (due to space constraints).  Plot PI as a function of tubing ID. Comment on your results.

Special tests on the oil reveal asphaltene precipitation at 1100 psi.  To avoid asphaltenes, you will need to size your wells to operate at 1200 psi outlet pressure (or higher).

3.- Build a PIPESIM model for Well 1 and plot the initial liquid production rate of qpi as a function of tubing ID.  Assume that all seven wells produce at the same flow rate.  Is the well capable of delivering qMax?

Hint: Utilize system analysis. Assume a large casing internal diameter that could accommodate all tubing IDs tested.

4.- Select casing and tubing sizes from the catalog.  Allow at least 1 in. clearance between tubing and casing walls.  Assume rwf to be equal to the casing OD.  Remember to update your PI to match your selected production casing.

5.- Study the effect of water cut on the total liquid production.  Comment on your results.

6.- Business unit decided that, due to spare gas capacity, the only artificial lift option is gas-lift.  Gas-lift should be modeled as a single-point injection 100 ft above the packer.  Calculate the required injected gas rate to meet the maximum production rate.  Plot total gas injected and wellhead temperature (WHT) as a function of water cut.  Are you able to meet your production targets while respecting all the constraints for every water cut?  Repeat the procedure until you do.  Please comment.

### II.- Pipeline design:

It is recommended to design the subsea pipeline separately from the wells to reduce the model complexity.  Pipeline separator pressure is set at 350 psi.

1.- What are the boundary conditions that you require to model the pipelines?

2.- From your answers on I.- 6.- gather boundary conditions required to design the pipelines.

Hint: Model the pipeline as a mass to pressure boundary. Assume all wells to behave the same as W1. The gas injected for gas-lift will flow through the pipeline. This can be modeled directly as GORPipeline = GORSat +Ginj/qpoil.

3.- Select a pipeline internal diameter to meet the production constraints.  Set a random insulation thickness to start your simulations.  The company desires to design a pipeline that can operate under reduced production conditions, with three wells going offline simultaneously.  Comment on your choice and show that it could operate with three wells offline.

4.- With your selected pipe diameter, does the pipeline inlet pressure remain constant? Comment.  How can you ensure the wellhead pressure remains at 1200 psi or above?  What effect could this have on the boundary conditions used to size the pipeline?

5.- Wax Appearance Temperature (WAT) is 100°F. Plot the outlet temperature as a function of the field water cut.  Comment and choose a limiting case.  Determine an insulation thickness that keeps the fluids hot enough for the worst-case scenario.

### III.- Production profile and short economic analysis:

In this section, for simplicity, it is allowed (and recommended) to work only with Well 1 and assume all other wells behave exactly the same.

1.- Do you need to run the field life in a network scenario for each year and water-cut if you assume maximum production throughout the field life-time? Comment.

2.- Compute oil, gas, and water production rate through time.  Also, compute the injected gas over time.

3.- Build a network model and run three cases with the entire system modeled for 5% WC, 40% WC, and 80%WC.  Show that your system is expected to operate properly, respecting all constraints, for all seven wells producing and for three wells offline and four wells producing.

Cost assumptions (expenses): Assume a water treatment cost of $1 per STB of water.  The gas injected is taken from the separator at 350 psi.  Assume that the compressor must boost the gas to the bottom-hole pressure psi to inject for gas-lift.  Assume gas injection cost of and 1 ($*d)/(MMScf*psi).

4.- Assume an oil sales price of 50 $/STB of oil. Calculate the critical water cut at which it is no longer economically viable to produce at the maximum liquid production rate.

5.- Do you need to stop operations at that water cut? Comment.

6.- Develop your own plan for production by modifying the gas injection plan.  Develop an equation or a method to estimate an optimal gas lift injection rate.

7.- With your new plan for gas lift injection, compute the production profile through time. Were you able to extend the field life and its economic performance? Comment.

8.- Do you see any potential issues with the pipeline sized in II? Propose ways to handle proper operations.

9.- Repeat this procedure for an Oil price of 30 $/STB.

Part 2

- Network coding (any programming language of your choice)

Assume that your PIPESIM license has expired, but you need to simulate your network (up to node m2). You will be asked to simulate a small network (which will be assigned).  Model boundary conditions from your wells as constant mass sources; you can take them from your PIPESIM simulations. Please write your own code for one of the scenarios studied in the main part. You can use solvers (optimizers) available in Python or MATLAB.

Please make the following assumptions:

- You can use your codes from homework assignments.

- Please assume Hagedorn & Brown correlation for vertical pipes and Beggs & Brill for horizontal pipes.  Assume the swap angle of 45o between correlations.

- Select any black oil correlations of your choice.

- Treat oil-water mixture as one fluid. Use appropriate correlations to calculate the properties of oil-water mixtures.

- Assume a constant temperature of 150°F.

- Make other necessary assumptions as needed.

You need to report:

- Discuss your code structure and the issues you encountered.

- Compare the profile plots for pressure obtained from your code and PIPESIM. Discuss how your temperature assumption affects the results.

- Run PIPESIM simulations of your network using different flow models (use at least 2 mechanistic models). Discuss how changing the flow model affects your results and the possible consequences.

- Wellbore – reservoir coding (any programming language of your choice) –

Assume that your PIPESIM license has expired, but you need to simulate the performance of one of your wells with no gas-lift (the well will be assigned).  Use constant reservoir pressure and constant wellhead pressure as your boundary conditions; you can take them from your PIPESIM simulations.  Please write your own code for one of the scenarios studied in the main part. You can use solvers (optimizers) available in Python or MATLAB.

Please make the following assumptions:

- You can use your codes from homework assignments.

- Please assume Hagedorn & Brown correlation for vertical pipes and Beggs & Brill for horizontal pipes.  Assume the swap angle of 45o between correlations.

- Use your original fluid.  You can use black oil correlations of your choice.

- Treat oil-water mixture as one fluid.  Use proper correlations to calculate oil-water mixture properties.

- Assume linear fluid temperature distribution.  Wellhead temperature is equal to 150°F.

- Make other necessary assumptions as needed.

You need to report:

- Discuss your code structure and the issues you encountered.

- Compare the profile plots for pressure/holdup for the nodal point obtained from your code and PIPESIM.  Discuss how your temperature assumption affects the results.

- Run PIPESIM simulations of using different flow models (at least two mechanistic models).  Discuss how changing the flow model affects your results and what are the possible consequences.

DESIGN REPORT FORMAT

| Table of Contents |  |
| --- | --- |
| Executive Summary |  |
| Introduction | Brief description of the project and objectives. Refer to the Appendix for a copy of the detailed project description (handout). |
| Procedure | Tell what you did. Consider dividing into subsections |
| Results | Divide into the same subsections as in Procedure. Refer to figures and tables for your discussions. |
| Conclusions | An expanded version of what appears in Executive Summary. |
| References |  |
| Appendices |  |
| Addendum | Letters from each person in the design group stating his or her contribution |